# Code review — pvdlowe v0.1.0

Repository: https://github.com/VarunMandy/pvdlowe (private) Reviewed at: commit a9c4b31, re-audited after the nucleation correction and the ml/ subpackage. Scope: 9,124 lines across 22 modules; 97 tests passing.

Re-audit delta. M1, M2 and M3 are all unchanged — none has been fixed, and none has worsened. Two items are added below: N1 covers the new ml/ subpackage, and adhesion_energy joins the over-long function list at 101 lines. The five items under "Worth preserving" have gained a sixth.

Permalinks below are pinned to a9c4b31 so they remain valid as the code moves. Because the repository is private, they resolve only for collaborators.

Findings are ordered by risk, not by module. Each states the evidence and a recommended action. Nothing here is a correctness bug in the physics — the validation suite covers that — but several items would cause trouble for whoever inherits the code.

## Repository map

| Path | Lines | Role |
| --- | --- | --- |
| pvdlowe/optics/ | 1,249 | TMM solver, stacks, standards integration |
| pvdlowe/materials/ | 1,303 | dispersion, metals, alloys, TCOs, glass |
| pvdlowe/screening/ | ~900 | elements, scoring, Pareto, candidates |
| pvdlowe/electrical/ | 671 | size effect, percolation, calibration |
| pvdlowe/optimize/ | 529 | thickness optimisation, sweeps, series |
| pvdlowe/doe/ | 637 | factorial designs, sputter rate model |
| pvdlowe/dft/ | 461 | VASP input generation |
| pvdlowe/mp/ | ~350 | Materials Project client and screening |
| pvdlowe/ml/ | 436 | ML interatomic potential surrogate (see N1) |
| pvdlowe/validate.py | 310 | model-vs-literature, consistency checks |
| tests/ | — | 97 tests, physics-level assertions |

Where to start reading: optics/stack.py for the central design object, then electrical/thinfilm.py for the coupling described in P2, then screening/scoring.py.

## Summary

| Severity | Count | Theme |
| --- | --- | --- |
| Medium | 3 | latent divergence, silent failure, dead abstraction |
| Low | 4 | API surface, function length, annotation gaps, import placement |
| Positive | 5 | worth preserving through any refactor |

No high-severity findings. The most consequential item (M1) is currently harmless and will not stay that way.

## M1 — Record construction is duplicated, and the copies are already divergent

Evidence. screening/candidates.evaluate() builds the canonical scoreable record. optimize/sweep.composition_series() builds its own:

r = performance_summary(coating)
r["cost_usd_per_m2"] = coating_material_cost(masses)["total_usd_per_m2"]
r["supply_risk"] = composition_supply_risk(coating.metal_alloy)

The hand-rolled version omits ten keys that evaluate() supplies: structural_stability, thermal_stability_c, deposition_efficiency, film_resistivity_uohm_cm, size_effect_ratio, metal_areal_mass_g_m2, mixing_model, provenance, reportable, id.

Why it is currently harmless. The three scored criteria among those (structural_stability, thermal_stability_c, deposition_efficiency) are all None, and ScoringScheme renormalises over available criteria. Both paths score an identical 64.95 for the benchmark. Verified.

Why it will not stay harmless. The moment any of those three is populated — which is the explicit intent, since they carry 0.30 of the weight in targets.yaml — the two paths will silently score on different criteria subsets. A composition series would then be non-comparable with the candidate table, and nothing would flag it. The same pattern was reproduced in several analysis scripts written during this project, which is how it propagates.

Recommended. Extract the record construction into one function, e.g. candidates.record_for(coating), and have both evaluate() and composition_series() call it. Add a test asserting the two paths produce identical key sets.

## M2 — The optimiser swallows all exceptions and returns a sentinel

Evidence. optimize/thickness.py#L98-L102

try:
    return -objective_value(build(metal, m, b, t, preset), scheme, criteria)
except Exception:
    return 1e3

Any failure — a bad dispersion lookup, a malformed preset, an arithmetic overflow — is indistinguishable from "this is a poor design". The optimiser will steer away from the region and report a converged result.

Compounding issue. The guard is not doing what it appears to. Sub-percolation coatings do not raise; they return finite scores:

metal 0.1 nm -> score 4.72
metal 1.0 nm -> score 4.53
metal 5.0 nm -> score 3.45

So the except branch is effectively unreachable in normal use, and the physical rejection it seems to provide is actually coming from the desirability functions scoring badly. That is fine, but it means the try/except is dead protection that would only ever fire on a genuine bug, which it would then hide.

Recommended. Catch specific exceptions, count them, and surface the count in the result dict. If more than a few per cent of evaluations fail, that is information the caller needs.

## M3 — _cache on the coating classes is dead

Evidence. LowECoating._cache and MultiMetalCoating._cache are declared as fields and reset in every replace() call:

return replace(self, metal_thickness_nm=float(thickness_nm), _cache={})

They are never read or written anywhere. Only Alloy._cache is genuinely used (alloys.py#L247-L268).

Why it matters. It advertises memoisation that does not exist. Someone optimising a hot loop would reasonably assume stack() is cached — it is not, and it rebuilds every dispersion on every call, which is the actual hot path in the sweeps. Meanwhile the _cache={} arguments are noise in three call sites.

Recommended. Either delete the field and the three _cache={} arguments, or implement the memoisation the field implies. The second is worth doing: the composition series calls stack() tens of thousands of times.

## N1 — The ml/ subpackage ships untested against a real backend

Evidence. pvdlowe/ml/surrogate.py is 436 lines and no line of it that touches an actual interatomic potential has ever executed. The development environment had no network, so mace-torch, chgnet and matgl could not be installed and the model weights could not be downloaded. The two tests that exist —test_mlip_surrogate_refuses_rather_than_falls_back and test_mlip_boundary_excludes_optical_properties — verify the absence path and the documented boundary. Neither verifies a calculation.

Why it is nonetheless shipped. The alternative was to omit the capability entirely. The module fails loudly with installation instructions when no backend is present, and the entry-point example gates every prediction behind reproducing two known Materials Project hull distances, so a silently wrong result is unlikely. But the honest status is written, syntax-checked, never run.

Specific risks that first execution will probably surface: the slab construction in adhesion_energy assumes SlabGenerator returns at least one slab for every proxy and takes the first without inspecting termination; the metal-film tiling uses a nearest-integer repeat that could leave several per cent lattice mismatch unreported as an error; and _e() uses a needlessly convoluted call form that should simply branch.

Recommended. Run examples/09_mlip_adhesion.py once on a machine with a backend installed, then either fix what breaks or mark the module experimental in docs/README.md. Do not cite any number from it until then.

## L1 — The top-level package exports almost nothing

pvdlowe/__init__.py#L12

>>> import pvdlowe; pvdlowe.__all__
['Provenance', 'Quantity', 'Record', '__version__']

Everything else requires a deep import (from pvdlowe.optics.stack import dmd). That is defensible for a library with a large surface, but it makes the first five minutes harder than necessary and the README examples all use deep paths.

Recommended. Re-export the dozen or so entry points people actually start with: dmd, dmdmd, LowECoating, MultiMetalCoating, performance_summary, ScoringScheme, evaluate_all.

## L2 — Several functions are too long to review comfortably

| Lines | Function | Source |
| --- | --- | --- |
| 112 | silver_reduction_curve | thickness.py#L158-L270 |
| 89 | solve | tmm.py#L73-L162 |
| 81 | composition_series | sweep.py#L112-L193 |
| 78 | build_parser | cli.py#L279-L357 |
| 101 | adhesion_energy | ml/surrogate.py — slab build, tiling, three energies and result assembly in one function; the clearest split candidate in the codebase |
| 87 | check_consistency | validate.py — three independent checks that could be three functions |
| 76 | fit_series | calibrate.py#L216-L292 |
| 72 | diagnose | calibrate.py#L295-L367 |
| 68 | cmd_calibrate | cli.py#L151-L219 |

build_parser is fine — argparse is inherently long and linear. solve is dense but cohesive. silver_reduction_curve is the one worth splitting: it contains a nested grid search, a Nelder–Mead refinement, a bisection, and result assembly, and the bisection logic in it was the source of a real bug during development (a conducting film was wrongly assumed to cap d_c).

## L3 — Return annotations at 86%

282 of 328 public functions annotated. The gaps are concentrated in cli.py (where it matters least) and in a few screening helpers (where it matters more, since they return heterogeneous dicts).

Recommended. TypedDict for the record and result dicts would document the contract that M1 is currently violating implicitly.

## L4 — Function-local imports are used inconsistently

Some are deliberate and correct — mp_api in client.py, avoiding a hard dependency; the four in sweep.py#L131-L134, avoiding a cycle. Others appear to be habit. A reader cannot tell which is which.

Recommended. A one-line comment on the deliberate ones, e.g. # local import: optional dependency or # local import: breaks a cycle.

## Worth preserving

These are the parts a refactor should not disturb.

P1 — The provenance system. Grading every quantity and refusing to admit HYPOTHESIS values into headline tables is unusual and it did real work: it is what kept the Ti-ternary predictions from being quoted as results. The tri-state verification flag (True/"partial"/"disputed") is a good refinement, though note it caused a bug when bool() collapsed it — now caught by test_validation_table_reports_the_true_source_state.

P2 — The optics/transport coupling. Film resistivity feeding back into Drude damping is the single most important design decision in the codebase. Without it, optimising for transmittance drives metal thickness to zero. Any refactor that decouples these will silently break the physics, and test_thin_film_penalised_in_both_optics_and_transport exists to catch that.

P3 — Tests that encode physics, not interfaces. test_energy_conservation, test_quarter_wave_antireflection, test_impedance_relation_round_trips and test_conductive_oxide_rejects_solar_near_infrared verify that the model obeys physical law rather than that functions return the right type. This is why the suite is worth running after any environment change.

P4 — Functions that refuse rather than guess. SputterModel.rate_nm_per_min raises when uncalibrated instead of returning a plausible number; MPClient raises when offline with a cold cache. Both were tempting to make "helpful". Both would have produced fabricated data.

P6 — The empirical parameter that carries its own provenance. TCOPreset.metal_growth_factor was added after measurement contradicted a model prediction. Its docstring states what the model got wrong, which paper corrected it, and that the values are calibrated for silver with copper untested. That is the right shape for an empirical fudge factor: not hidden, not defended, and explicit about the domain it was fitted on.

P5 — Docstrings that record why, not what. Several encode reasoning that would otherwise be lost — the Rakić damping re-anchoring, the p/R non-identifiability, the trilayer de-embedding non-linearity. These are the highest-value comments in the codebase.

## Recommended order of work

- N1 — run the ml/ subpackage once against a real backend. It is the only part of the codebase with no execution evidence at all.

- M1 — extract the shared record builder. Half an hour, prevents a silent correctness failure the moment the stability criteria are populated.

- M3 — delete or implement _cache. Deleting is ten minutes; implementing would measurably speed the sweeps.

- M2 — narrow the exception handling and report the failure count.

- L1 — top-level re-exports, for whoever picks this up.

- L2–L4 as opportunity allows.

None of these blocks handover. M1 is the only one that will cause a wrong number, and only after someone populates a criterion that is currently None.